PARAMETERIZED ALU

DESIGN & VERIFICATION REPORT

Submitted by

Tanishq

Batch 11

FE Tools Training Program

May 2026

# 1. Project Introduction

This project presents the design and functional verification of a parameterized 8-bit Arithmetic Logic Unit (ALU) implemented in synthesizable Verilog. The ALU supports a comprehensive set of arithmetic and logical operations, organized under two operating modes, controlled via a single Mode bit.

The design is synchronous, operating on the rising edge of a clock (CLK), with asynchronous active-high reset (rst) and clock-enable (ce) support. Operands are registered internally on the first clock cycle, and results are produced on subsequent cycles — making the ALU a pipelined, multi-cycle design for certain complex operations.

The parameterized nature of the design (data_width) allows the ALU to be scaled from 2-bit to n-bit wordlengths without any modification to the RTL code, enabling reuse across different system configurations.

Verification is accomplished through a self-checking SystemVerilog testbench that computes expected outputs in parallel with the DUT and compares results after appropriate latency, using a structured test matrix of 83 test cases covering valid operations, invalid operand combinations, corner cases, and pipeline stress scenarios.

# 2. Objectives

- Study and understand the architectural requirements for a multi-mode, pipelined ALU.

- Design a parameterized, synthesizable Verilog RTL implementation supporting 14 arithmetic and 14 logical operations.

- Implement proper pipeline stages for multi-cycle operations (incr_mult, shift_mult) with a cnt-based controller.

- Develop a comprehensive self-checking SystemVerilog testbench with parallel expected-value computation.

- Verify correctness across normal cases, edge cases, invalid INP_valid encodings, and mid-operation signal changes.

- Achieve maximum code and functional coverage using Questa SIM's coverage reporting tools.

- Analyze and document the ALU's timing behaviour, operation semantics, and verification results.

# 3. Design Architecture

The ALU is implemented as a single Verilog module with a configurable data width parameter. Internally, it is organized into three always blocks: an input registration block, a pipeline controller block, and a combinational logic block.

## 3.1 Module Ports

| Signal | Width | Description |
| --- | --- | --- |
| opa | [N-1:0] | Operand A – primary arithmetic/logic input |
| opb | [N-1:0] | Operand B – secondary input or shift/rotate amount |
| cin | 1 | Carry-in for add_cin and sub_cin operations |
| CLK | 1 | System clock – rising edge triggered |
| rst | 1 | Asynchronous active-high reset |
| ce | 1 | Clock enable – freezes pipeline when deasserted |
| Mode | 1 | 0 = Logical operations; 1 = Arithmetic operations |
| INP_valid | [1:0] | Operand validity: 2'b01=A only, 2'b10=B only, 2'b11=Both |
| Cmd | [3:0] | 4-bit operation select code (0–12 valid, Mode-dependent) |

### Output Signals

| Signal | Width | Description |
| --- | --- | --- |
| res | [2N-1:0] | Result register – double-width to accommodate multiplication |
| oflow | 1 | Overflow flag for signed/unsigned arithmetic |
| cout | 1 | Carry-out flag for unsigned addition |
| g / l / e | 1 each | Greater-than, Less-than, Equal-to flags for comparator operation |
| err | 1 | Error flag: asserted on invalid INP_valid or unsupported Cmd |

# 4. Timing Behaviour

The ALU uses a two-stage pipeline for most operations and a three-stage pipeline for the two multiplication operations (incr_mult, shift_mult).

| Operation Class | Latency (Cycles) | Notes |
| --- | --- | --- |
| All Logical Operations | 2 | Inputs sampled at cycle 1, result at cycle 2 |
| Most Arithmetic Ops | 2 | add, sub, inc, dec, cmp, signed_add/sub |
| incr_mult / shift_mult | 3 | cnt counts 0→1→2; result registered at cnt==2 |

The cnt register is incremented by the pipeline controller on each clock edge when ce is high and a multi-cycle operation is in progress. When cnt reaches 2, the computed process_r value is latched into res and cnt resets to 0.

The ce (clock enable) signal freezes the entire pipeline when deasserted — no new inputs are sampled and no outputs are updated. This behaviour was verified by the TC_41 and TC_48 mid-operation CE toggle tests.

# 5. Supported Operations

## 5.1 Logical Operations (Mode = 0)

| Cmd | Operation | INP_valid | Description |
| --- | --- | --- | --- |
| 0 | AND | 11 | Bitwise AND of opa and opb |
| 1 | NAND | 11 | Bitwise NAND of opa and opb |
| 2 | OR | 11 | Bitwise OR |
| 3 | NOR | 11 | Bitwise NOR |
| 4 | XOR | 11 | Bitwise XOR |
| 5 | XNOR | 11 | Bitwise XNOR |
| 6 | NOT A | 01/11 | Bitwise NOT of opa |
| 7 | NOT B | 10/11 | Bitwise NOT of opb |
| 8 | SHL A | 01/11 | Shift opa left by 1 bit |
| 9 | SHL B | 10/11 | Shift opb left by 1 bit |
| 10 | SHR A | 01/11 | Shift opa right by 1 bit (logical) |
| 11 | SHR B | 10/11 | Shift opb right by 1 bit (logical) |
| 12 | ROL A | 11 | Rotate opa left by opb[2:0] positions; err if opb[N-1:3]!=0 |
| 13 | ROR A | 11 | Rotate opa right by opb[2:0] positions; err if opb[N-1:3]!=0 |

## 5.2 Arithmetic Operations (Mode = 1)

| Cmd | Operation | INP_valid | Description |
| --- | --- | --- | --- |
| 0 | ADD | 11 | Unsigned add; cout on carry-out |
| 1 | SUB | 11 | Unsigned subtract; oflow when opa < opb |
| 2 | ADD with CIN | 11 | opa + opb + cin; cout on carry-out |
| 3 | SUB with CIN | 11 | opa - opb - cin; oflow on underflow |
| 4 | INC A | 01/11 | Increment opa by 1 |
| 5 | DEC A | 01/11 | Decrement opa by 1 |
| 6 | INC B | 10/11 | Increment opb by 1 |
| 7 | DEC B | 10/11 | Decrement opb by 1 |
| 8 | CMP | 11 | Compare: sets g, l, e flags |
| 9 | INCR_MULT | 11 | (opa+1)*(opb+1); 3-cycle pipeline |
| 10 | SHIFT_MULT | 11 | (opa<<1)*opb; 3-cycle pipeline |
| 11 | SIGNED ADD | 11 | $signed add; overflow on sign bit change |
| 12 | SIGNED SUB | 11 | $signed sub; overflow on sign bit change |

# 6. Working of the Design

## Phase 1: Input Registration

On every rising clock edge where ce is asserted and the pipeline is not busy (cnt != 1), the primary operands opa, opb, and INP_valid are latched into internal registers s_opa, s_opb, and inp_valid respectively. This prevents operand glitching during long operations.

## Phase 2: Combinational Operation

The always@(*) combinational block evaluates the registered inputs based on the current Mode and Cmd. For single-cycle operations the result is placed directly in t_res (logical) or b_res (arithmetic). For multi-cycle multiply operations, the intermediate result is stored in process_r.

The INP_valid field is checked for every operation: two-operand instructions require 2'b11, while single-operand instructions accept 2'b01 or 2'b10 in addition to 2'b11. Any mismatch sets t_err, and unsupported Cmd values fall through to a default case that also sets t_err.

## Phase 3: Output Registration

The pipeline controller always block manages the cnt counter and output latching. For single-cycle ops, outputs are registered every cycle when ce is high. For incr_mult and shift_mult, the cnt counter increments until it reaches 2, at which point process_r is committed to res and cnt resets to 0.

# 7. Testbench Architecture

The testbench (tb_alu) is a self-checking, directed testbench written in SystemVerilog. It instantiates the DUT and drives stimuli through two task types: do_op for standard single/two-cycle operations, and test_mid_op_change for pipeline stress testing.

| Component | Description |
| --- | --- |
| do_op task | Drives Mode/Cmd/INP_valid/opa/opb/cin; waits appropriate cycles; calls check_outputs |
| compute_expected | Parallel golden model mirroring DUT logic; sets exp_res, exp_err, exp_oflow, exp_cout, exp_g/l/e |
| check_outputs | Compares DUT outputs against expected; prints [PASS]/[FAIL] with full signal dumps on mismatch |
| test_mid_op_change | Tests 4 pipeline-stress scenarios: Mode change, Cmd change, Operand change, CE deassert mid-flight |
| Clock Generator | 10 ns period (5 ns half-period); free-running forever loop |
| Scoreboard | error_count and test_count integers; final summary printed at end of simulation |

# 8. Testbench Components

## 8.1 Stimulus Synchronization

All inputs are driven on negedge CLK to ensure setup-time compliance with the DUT's posedge-triggered flops. The testbench waits for the appropriate number of posedge CLK edges after driving inputs before sampling outputs on the next negedge.

## 8.2 INP_valid Encoding

- 2'b00 — No operand valid; all two-operand and single-operand commands assert err.

- 2'b01 — Only opa valid; two-operand commands assert err; opa-only commands proceed.

- 2'b10 — Only opb valid; two-operand commands assert err; opb-only commands proceed.

- 2'b11 — Both operands valid; all commands proceed normally.

## 8.3 Mid-Operation Change Tests (TC 38–41, 45–48)

These eight tests use test_mid_op_change to verify pipeline robustness:

- Type 1 — Mode change after cycle 1: verifies Mode is captured from registered inputs, not combinational.

- Type 2 — Cmd change after cycle 1: verifies Cmd does not corrupt an in-flight operation.

- Type 3 — Operand change after cycle 1: verifies s_opa/s_opb isolation from combinational opa/opb changes.

- Type 4 — CE deassert mid-flight: freezes pipeline for 3 cycles, then resumes; output must remain unchanged.

# 9. Quality of Code Assessment

## 9.1 RTL Lint Analysis

The design was reviewed for common lint violations before simulation:

- No combinational feedback loops detected — all outputs are registered.

- All case statements include a default branch to avoid latches and handle out-of-range Cmd codes.

- Blocking assignments used only in combinational always@(*) blocks; non-blocking assignments used in clocked always blocks — no mixed assignment violations.

- The double-width res output (2*data_width) correctly prevents truncation of multiplication results.

- Parameterization is fully propagated — no hardcoded bit-widths inside the module body.

## 9.2 Code Coverage

Coverage was collected using Questa SIM's automatic instrumentation. The following coverage types were tracked: Statement, Branch, FEC Expression, FEC Condition, and Toggle coverage.

# 10. Simulation Results

Simulation was performed in Questa SIM. The testbench executed a matrix of 81 directed test cases covering all supported operations, invalid operand scenarios, corner cases, and pipeline stress events.

| Metric | Value |
| --- | --- |
| Total Test Cases Executed | 81 / 83 |
| Tests Passed | 81 |
| Tests Failed | 0 |
| Simulation Tool | Questa SIM |
| RTL Design Tool | Vivado (synthesis check) |
| Clock Period | 10 ns |
| Data Width | 8 bits (parameterized) |

## 10.1 Test Case Coverage Summary

| TC Range | Category | Count | Result |
| --- | --- | --- | --- |
| TC 07–13 | General / Configuration (Mode, CE, INP_valid, invalid Cmd) | 7 | ALL PASS |
| TC 14–55 | Arithmetic Operations (add, sub, inc, dec, cmp, mult, signed) | 42 | ALL PASS |
| TC 56–83 | Logical Operations (AND/OR/XOR/NOT/SHL/SHR/ROL/ROR and invalids) | 28 | ALL PASS |
| TC 38–41, 45–48 | Mid-operation pipeline stress tests (incl. CE toggle) | 8 | ALL PASS |

## 10.2 Notable Corner Cases Verified

- TC_16: ADD with opa=0xFF, opb=0x01 — verifies cout assertion on carry-out.

- TC_19: SUB with opa=0x00, opb=0x01 — verifies oflow assertion on unsigned underflow.

- TC_25: SUB_CIN with opa=0x00, opb=0x00, cin=1 — verifies underflow with borrow.

- TC_42: INCR_MULT with opa=0xFF, opb=0xFF — (256*256=65536) verifies 16-bit result correctness.

- TC_49: SHIFT_MULT with opa=0x80 — (0x80<<1)=0x00 for 8-bit; verifies overflow behavior.

- TC_51: SIGNED ADD 0x7F + 0x01 = 0x80 — verifies positive overflow detection.

- TC_54: SIGNED SUB 0x80 - 0x01 = 0x7F — verifies negative overflow detection.

- TC_80/83: ROL/ROR with opb=0x08 — shift amount ≥ 8 triggers err flag correctly.

# 11. Coverage Report

Coverage was collected and analyzed in Questa SIM. The report was generated after the full test matrix execution and is shown below.

## 11.1 Coverage Screenshot

Figure: Questa SIM Coverage Summary Report

## 11.2 Coverage Observations

| Coverage Type | Bins | Hits | Misses | % Hit | Coverage |
| --- | --- | --- | --- | --- | --- |
| Statements | 106 | 106 | 0 | 100% | 100.00% |
| Branches | 99 | 99 | 0 | 100% | 100.00% |
| FEC Expressions | 4 | 4 | 0 | 100% | 100.00% |
| FEC Conditions | 27 | 27 | 0 | 100% | 100.00% |
| Toggles | 278 | 268 | 10 | 96.40% | 96.40% |
| TOTAL | — | — | 10 | — | 99.28% |

## 11.3 Analysis

- Statement, Branch, FEC Expression, and FEC Condition coverages all achieved 100%, confirming that every line of RTL code was exercised and every conditional branch was traversed in both directions.

- Toggle coverage reached 96.40%, with 10 toggle bins remaining uncovered. These 10 misses are attributable to the rotate_left and rotate_right function outputs — which achieve 100% statement/branch coverage individually but may have specific bit-toggle combinations that require exhaustive random stimuli to exercise fully.

- Overall DUT coverage of 99.28% is an excellent result for a directed testbench, indicating thorough functional verification with minimal gaps.

- The rotate functions (rotate_left, rotate_right) show 100% statement and branch coverage with toggle metrics marked '--' (not applicable at function scope level), consistent with the tool's reporting for synthesizable functions.

# 12. Conclusion

This project successfully demonstrated the complete RTL design and functional verification flow for a parameterized 8-bit ALU. The design supports 28 distinct operations across two modes, handles invalid operand combinations gracefully via the err flag, and implements a correct pipelined architecture for multi-cycle multiplication operations.

The self-checking testbench executed 81 directed test cases with zero failures, achieving 99.28% total functional coverage in Questa SIM. All statements (100%), branches (100%), FEC expressions (100%), and FEC conditions (100%) were covered. The 3.6% toggle miss is within acceptable bounds for a directed test strategy and is localized to specific bit transitions in the rotate path.

The project reinforced key RTL design principles: parameterization for reusability, registered inputs for pipeline stability, clock-enable for power-aware freezing, and always-default case branches for robust error handling.

# 13. Future Work

- Extend the testbench with constrained-random stimulus (using SystemVerilog classes and covergroups) to close the remaining 3.6% toggle coverage gap.

- Add formal property verification (SVA assertions) to prove the pipeline protocol, INP_valid semantics, and signed overflow conditions exhaustively.

- Scale the parameterized design to 16-bit and 32-bit widths and re-run the full test matrix to validate width-independence.

- Integrate a UVM-based verification environment with a reference model, functional coverage groups, and automated regression for production-grade sign-off.

- Synthesize the design in Vivado targeting a Xilinx FPGA and measure timing closure, LUT utilization, and maximum operating frequency.

- Add barrel-shifter support for multi-bit shift/rotate operations (currently shift by only 1 bit is supported for non-rotate operations).

--- End of Report ---